Checking the frequency drift of a receiver or transceiver.

By KB0HAE

Having written a few emails about this subject in the last couple of years, I decided to create this document to more easily share this method of checking frequency drift in transceivers and receivers.

What you need:

A transceiver or receiver to test of course.

A computer with a sound card.

A way to get audio from the radio to the sound card.  See the text below.

A PSK31 program.  I use Digipan.

You can measure the amount of drift that you have in the following manner: 

If you are set up for PSK or other digital mode operation. connect your interface.  If not, you can connect a cable from the headphone jack on the radio to the line in on your computer sound card, or even just place mic connected to the computer sound card near the radio speaker.   I have also placed an ear bud (connected to the headphone jack of the radio) next to the mic on my ancient laptop (this would probably work with about any mic connected to the sound card of any computer).

Tune the radio to WWV, and start your PSK software.  I use Digipan. Tune the VFO so that you see the WWV carrier on the waterfall display.  You may have to adjust the mixer input controls on the computer, and the AF gain (volume) on the radio.  Click on the center of the carrier, and write down the time and the audio frequency shown in the program.  Check again at 15 minute intervals, and write down the time and audio frequency.  The difference in the audio frequency is the amount of drift.  After about 20 minutes or so, the drift should be minimal(50Hz or less in any 15 minute period after warm-up) 

Since WWV is a frequency standard, what you are seeing is the VFO drift (and and drift in the sound card if any).

I have used this method for years.  It is pretty accurate. How do I know its accurate? Well, the results  agree with the results measuring transmit frequency with a frequency counter.   A frequency counter is not always available.   My Kenwood TS-530S drifts about 100 HZ per half hour after warm-up.  My Kenwood TS-130SE drifts about 29Hz per hour, which is very good for a radio with a mechanical VFO (not PLL synthesized frequency control).

Older radios with mechanical VFOs will experience frequency drift.  In any of the older Kenwood radios that I have owned, this has always been within the radio's specs, and has not been a problem for PSK31/SSTV/digital mode operations.